# How to Ansys

## Downloading ANSYS

Link to Student Ansys download

- There are a few different free versions of Ansys, but this is the most versatile and will probably suit your needs just fine.

- Make sure you have enough space on your computer beforehand, or it won’t finish the download.

- After download, you want to open the Ansys Workbench. If you have some trouble finding it, this forum question might help.

### Subheading

You may or may not need this, but here it is just in case.

- If you’re listing steps

- to do something,

- then maybe using a

- numbered list

- might be helpful!

  - Maybe even substeps too…

## Random Notes to be organized later

- Start from the Workbench

- Fluid Flow → Geometry → DesignModeler

  - C-shape domain converges better than an oval-shaped domain

  - Create domain in a sketch → turn sketch to lines → turn lines into surface → turn solid surface into fluid surface

    - This surface represents the fluid surface

  - Need to split domain into faces to make the mesh software work easier

- Workbench → Mesh → Mechanical

  - Want smaller edges near the airfoil (subject)

  - First cell height thickness

    - First cell height at a y+ of 1 (to capture nonlinearity of flow near the surface)

    - Need to pick a Reynolds number for the flow

    - Re → mainstream velocity [m/s] → use this link to get y+ (shown as delta s here)

    - If your cell height is less than this value, you’re good

    - This will vary across the wall; make sure you check it after the simulation

  - Boundary conditions

- Setup

  - Make Polyhedral has better convergence quality

- Video on airfoils

- Video on 3D geometry

- An easier video on 3D geometry