# How to design a drivetrain

This document covers:

- How to choose a design

- Determining belt lengths

- Designing drive pulleys

- Choosing a drive motor and gearbox combo

# Drive System Design Guide

Drive Calculation Spreadsheet: https://bit.ly/JustCuzDriveCalc

For the calculator you need to know the # of wheels, # of motors, wheel diameter, and motor specs

### Main parameters that determine the needed motor/gearing

- Weight class (how much weight can be dedicated to drive?)

- Drive Strategy - control, wedge, or other?

  - Speed is actually a lower priority for a control bot than a vertical spinner - spinners need to achieve good bite and escape from a bad position

  - Pushing power/low end torque are critical for control bots, especially w/o wedges

  - Generally a wedge, lifter, grabber, or other control bot will want more drive power than a spinner

- Determining how much drive power is needed is a function of the desired speed and torque for your chosen wheels

- According to Ask Aaron’s drivetrain guidelines, you will want at least 1.5-2X the torque required to spin the robot wheels to prevent stalling/overloading the motor while pushing an opponent into or against a wall.

  - Note: More torque with a reasonable top speed >> stupid fast top speed

  - Having the ability to go 100mph if you only have room to accelerate to 20mph means leaving lots of torque on the table

  - Combat arenas are small - acceleration and pushing power is often far more important than speed, and both are only related to torque and traction

### Transferring Power to Wheels - Mechanics and Reductions

Direct drive:

Prosoften risky and fragile, but very simple; no external gearing

  - Supporting the other end of the axle is recommended - easy to destroy the motor with impacts

  - Basically just don't even think about it with an ungeared motor unless you have a very special circumstance (ultra low kV brushless yolodrive???)

Belt/chain drive

Simplest/easiest way to run 4WD off 2 motors, no tensioning needed if done properly

  - Generally easy to attach pulleys to a motor shaft

  - Typically can get no more than 3:1 per stage

  - Timing belts or chains are often best for 12lb and up as they won't slip, but round belts work great for smaller bots with lower torque

  - I managed to use O rings in place of welding my own PU belts in Draconid given sufficient stretch/tension, but this can be difficult to size properly

  - Note: all plain rubber belts lose significant tension over time

  - 3D printing pulleys for belts works extremely well

Planetary Gearboxes - Pricey but compact and robust

  - Not good with impact loads, but often beefy output shafts that easily handle shear loads (best to support the axle if you can though)

  - Handle a ton of torque and very compact for the high ratios

  - Extremely expensive to get many P60s or Versas, expect $50-90 per drive side in a 12lb+ bot

  - Often needs an adapter to use brushless, but usually someone has done it before

  - Can drive a central pulley and belt to 4WD, or drive one wheel directly and belt to second for extra redundancy (see #5)

Gears

  - Metal gears are expensive but 3D printing is a great option (see Draconid)

  - Less impact tolerant than belts, but can be designed/made to spec

  - Pretty compact, but very restrictive on center-center

  - Unlike with pulleys/chains, it's not always possible to change ratios without changing center-center

  - 3D printed herringbone gears have a larger root area, stronger tooth profile

  - Need 3 gears per side to reverse direction for 2 motor 4WD

  - Some guesswork can be involved in determining the width and pitch/module needed

Combinations of the above!

  - No harm in combining two different approaches, as long as you’re careful

  - Can get the best of both worlds by combining gears with belts/chains - high reduction with shock isolation

  - It can be much cheaper to replace a shaft and pulley than a two stage gearbox

Where to Buy Mechanical Components

  - You can buy Chain Sprockets, Gears, Pulleys, wheels and more at Andymark, Vex, ServoCity, McMaster, and more!

  - P60/P80/P61 gearboxes are at Banebots, and Versaplanetary is Vex.

  - Wheels are available from tons of hobby stores, Amazon, Fingertech, Robotshop, several of the above vendors, etc.

    - Also take a look on Ebay, Banggood, or other Chinese retailers but YMMV

  - Brushless motors are available on Amazon, Hobbyking, Banggood, and any drone/RC hobby store

### Wheels, wheels, wheels!

- Softer rubber is more grippy but less durable

- Foam tires are able to take direct hits without trashing your bot unlike rubber (see: D2 kit wheels, Fingertech wheels, lite flights)

- Custom silicone cast tires are soft and grippy and often the best performing, but require a lot of experimentation, which means a lot of $$$ and time

- Be sure to think about how to actually transmit torque to the wheels

  - Live shaft turning with the wheel

    - Set screws on flats (Fingertech hubs)

    - Keyed hubs (Banebots and similar, common for larger bots)

    - D or Hex shaft/bore, or spline (also Banebots, VEX stuff, Servocity hubs)

  - Dead shaft - axle doesn’t spin

    - Bolt through to pulley/gear (Conduit)

    - Custom 3D printed things (Draconid)

Sources:

Team Tentacle torque Calculator: http://runamok.tech/squid/newtorquecalc.htm

AskAaron Drivetrain Gear Ratios Article: http://runamok.tech/AskAaron/optimum.html

Brushless motor torque estimation article

## Determining belt lengths

Required Prerequisite:

- Center to Center Distance Between Drive Pulleys

- Estimate of Drive Pulley Diameter

  - Later, will need Final Drive Pulley Diameter

## Designing drive pulleys

## Choosing a drive motor and gearbox combo

Use this drivetrain speccer template

(If you’re designing a spinning weapon, can also use this template)

### Subheading

You may or may not need this, but here it is just in case.

### Equation Derivations

You may or may not need this, but here it is just in case.

Time to top speed (assuming perfect grip) and final speed calculations

Given:

Pushing Force F [lbf]

Bot Mass m [lbm]

Max velocity vmax [mph]

Solve:

For velocity curve over time: v=at

tend=vmaxa

## Designing cleats

## Designing a walker

## Recommended materials