# How to bend titanium wedges

## Notes about titanium bending

- In order to achieve a certain bend angle, you have to overshoot your goal angle by about 15°. From there, the titanium will bend backward to the spot that you want it to be in.

- Titanium will break if you try to bend it to about 45°. Be sure to follow the procedure below carefully.

- The procedure outlined in Step 4 below will increase the bend radius, therefore making the plate less susceptible to breaking. (Bigger radius = less stress concentration—remember statics?)

## Procedure

- All sheet metal bending is done using the sheet metal bender in the Emerson machine shop, which is to your left when you enter the shop.

- Clamp your titanium plate onto the machine.

- For bends smaller than 45°:

  - Bend to whatever angle you want + about 15 extra degrees.

  - Done!

- For bends at or greater than 45°:

  - Bend titanium to no more than 55°.

  - Unclamp and pull the titanium out a few millimeters.

  - Re-clampe the plate and bend again.

  - Repeat until the desired angle is achieved.

  - Done!